BAOFENG BF-88ST Programming Guide

Contents

1. Cable Driver Installation	2

2. Radio Reading	3

3. Channel Information	5

4. Optional Feature	6

5. Write and Save	11

## 1. Cable Driver Installation

2-Pin K connector programming cable (package not included)

Compatible System

Latest Windows system (i.e. Windows 7, Windows 10)

Cable Driver and Guideline
If the following picture is not clear, please check the following link
https://cdn.shopify.com/s/files/1/0011/7220/9721/files/RD-201_Cable_Driver_Installation.png?v=1576652703

Download the corresponding driver which matches your computer system (Win7/Win8/Win10). They are available on the support section of baofengradio.com.

## 2. Radio Reading

Download and run the ‘BAOFENG Computer Software’ programming software, click Setting – Port menu, select the corresponding port number, click “OK”.

Read the current information from the radio to your PC to create an initial program template. Click Program – Read Data From Radio, or simply click the  icon.

## 3. Channel Information

The following is an introduction to each term.

| Name | Meaning | Setting | Description |
| --- | --- | --- | --- |
| RX QT/DQT DCS/CTS Decoder | Receiving CTCSS/DCS | Refer to the DCS table and CTCSS table in the manual. | Mutes the speaker of the transceiver in the absence of a specific low level digital signal. If the station you are listening to does not transmit this specific signal, you will not hear anything. |
| TX QT/DQT DCS/CTS Encoder | Transmitting CTCSS/DCS | Refer to the DCS table and CTCSS table in the manual. | Transmits a specific low-level digital signal to unlock the squelch of a distant receiver (usually a repeater). |

- Mode--- Function Setting

SQL: Squelch Level--0-9, Mutes the speaker of the transceiver in the absence of a strong signal. Squelch is either 0 or 1 - 9 levels. The higher level, the stronger the signal must be to in-mute the speaker.

TOT: Transmission time-out timer---30-300s. This feature provides a limits transmission time to a programmed value. This will promote battery conservation by not allowing you to make excessively long-time transmissions and in the event of a stuck PTT switch, it can prevent interference to other users as well as battery depletion.

Voice  Swtich：Voice  Swtich ON/OFF

Voice  Select：Voice for English/ Chinese

VOX: Level--1-5. When enabled it is not necessary to push the [PTT] button on the transceiver. Adjust the gain level to an appropriate sensitivity to allow smooth transmission.

- Write and Save

## Click Program - Write Data To Radio, or click the  icon to write and save the setting to the radio.